SYSTEM USED:-

- Ubuntu 20.04 (Focal)

- Noetic

- Gazebo 11.15.1

We will proceed in three clear phases, exactly as you requested:

- Room (static world)

- Biped robot

- Moving block / obstacle

Each phase will be:

- Minimal

- Reproducible

- Gazebo-native

- Thesis-safe

# Phase 1 — Room (Gazebo world)

## 1. Create a Catkin workspace (ROS 1)

mkdir -p ~/catkin_ws/src

cd ~/catkin_ws

catkin_make

source devel/setup.bash

(Optional: add source ~/catkin_ws/devel/setup.bash to ~/.bashrc)

## 2. Create a simulation package

cd ~/catkin_ws/src

catkin_create_pkg biped_simulation rospy roscpp gazebo_ros

## 3. Create directory structure

cd ~/catkin_ws/src/biped_simulation

mkdir -p worlds launch models

Result:

biped_simulation/

├── launch/

├── models/

├── worlds/

├── package.xml

└── CMakeLists.txt

## 4. Create the room world file

nano worlds/room.world

Paste exactly this:

<?xml version="1.0" ?>

<sdf version="1.6">

<world name="room">

<include>

<uri>model://sun</uri>

</include>

<include>

<uri>model://ground_plane</uri>

</include>

<!-- Walls -->

<model name="walls">

<static>true</static>

<link name="walls_link">

<!-- North wall -->

<collision name="north_collision">

<pose>0 5 1 0 0 0</pose>

<geometry>

<box><size>10 0.1 2</size></box>

</geometry>

</collision>

<visual name="north_visual">

<pose>0 5 1 0 0 0</pose>

<geometry>

<box><size>10 0.1 2</size></box>

</geometry>

</visual>

<!-- South wall -->

<collision name="south_collision">

<pose>0 -5 1 0 0 0</pose>

<geometry>

<box><size>10 0.1 2</size></box>

</geometry>

</collision>

<visual name="south_visual">

<pose>0 -5 1 0 0 0</pose>

<geometry>

<box><size>10 0.1 2</size></box>

</geometry>

</visual>

<!-- East wall -->

<collision name="east_collision">

<pose>5 0 1 0 0 0</pose>

<geometry>

<box><size>0.1 10 2</size></box>

</geometry>

</collision>

<visual name="east_visual">

<pose>5 0 1 0 0 0</pose>

<geometry>

<box><size>0.1 10 2</size></box>

</geometry>

</visual>

<!-- West wall -->

<collision name="west_collision">

<pose>-5 0 1 0 0 0</pose>

<geometry>

<box><size>0.1 10 2</size></box>

</geometry>

</collision>

<visual name="west_visual">

<pose>-5 0 1 0 0 0</pose>

<geometry>

<box><size>0.1 10 2</size></box>

</geometry>

</visual>

</link>

</model>

</world>

</sdf>

## 5. Create Gazebo launch file

nano launch/room.launch

<launch>

<include file="$(find gazebo_ros)/launch/empty_world.launch">

<arg name="world_name" value="$(find biped_simulation)/worlds/room.world"/>

<arg name="paused" value="true"/>

<arg name="use_sim_time" value="true"/>

<arg name="gui" value="true"/>

</include>

</launch>

## 6. Build the workspace

cd ~/catkin_ws

catkin_make

source devel/setup.bash

## 7. Launch the room

roslaunch biped_simulation room.launch

### Phase 2 — Biped robot

We will:

- Create a minimal URDF

- Spawn it into the room

- Add joints & controllers incrementally

We will start minimal and correct, then grow it.
First goal: spawn a simple biped standing in the room (no controllers yet).

## What we will build first (VERY important)

Minimal biped structure:

- Base (pelvis)

- Left leg: thigh → shin → foot

- Right leg: thigh → shin → foot

- Revolute joints only

- Correct inertias (Gazebo will explode without them)

👉 No walking yet
👉 No controllers yet
👉 Just visual + physics correctness

## 1. Create robot folders

cd ~/catkin_ws/src/biped_simulation

mkdir -p urdf launch

## 2. Create yaml

nano config/controllers.yaml

joint_state_controller:

type: joint_state_controller/JointStateController

publish_rate: 50

left_hip_controller:

type: effort_controllers/JointPositionController

joint: left_hip

pid: {p: 200, i: 0.0, d: 10}

left_knee_controller:

type: effort_controllers/JointPositionController

joint: left_knee

pid: {p: 300, i: 0.0, d: 15}

left_ankle_controller:

type: effort_controllers/JointPositionController

joint: left_ankle

pid: {p: 100, i: 0.0, d: 5}

right_hip_controller:

type: effort_controllers/JointPositionController

joint: right_hip

pid: {p: 200, i: 0.0, d: 10}

right_knee_controller:

type: effort_controllers/JointPositionController

joint: right_knee

pid: {p: 300, i: 0.0, d: 15}

right_ankle_controller:

type: effort_controllers/JointPositionController

joint: right_ankle

pid: {p: 100, i: 0.0, d: 5}

## 3. Create biped URDF (minimal)

nano urdf/biped.urdf

Paste exactly this:

<?xml version="1.0"?>

<robot name="biped">

<!-- ================= BASE ================= -->

<link name="pelvis">

<inertial>

<origin xyz="0 0 0"/>

<mass value="10"/>

<inertia ixx="0.1" iyy="0.1" izz="0.1"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<box size="0.3 0.2 0.15"/>

</geometry>

</visual>

<collision>

<geometry>

<box size="0.3 0.2 0.15"/>

</geometry>

</collision>

</link>

<!-- ================= LEFT LEG ================= -->

<link name="left_thigh">

<inertial>

<mass value="3"/>

<inertia ixx="0.05" iyy="0.05" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<cylinder radius="0.05" length="0.4"/>

</geometry>

</visual>

<collision>

<geometry>

<cylinder radius="0.05" length="0.4"/>

</geometry>

</collision>

</link>

<joint name="left_hip" type="revolute">

<parent link="pelvis"/>

<child link="left_thigh"/>

<origin xyz="0 0.15 -0.075"/>

<axis xyz="1 0 0"/>

<limit lower="-1.0" upper="1.0" effort="50" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<link name="left_shin">

<inertial>

<mass value="2"/>

<inertia ixx="0.04" iyy="0.04" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<cylinder radius="0.04" length="0.4"/>

</geometry>

</visual>

<collision>

<geometry>

<cylinder radius="0.04" length="0.4"/>

</geometry>

</collision>

</link>

<joint name="left_knee" type="revolute">

<parent link="left_thigh"/>

<child link="left_shin"/>

<origin xyz="0 0 -0.4"/>

<axis xyz="1 0 0"/>

<limit lower="0" upper="1.5" effort="50" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<link name="left_foot">

<inertial>

<mass value="1"/>

<inertia ixx="0.01" iyy="0.01" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<box size="0.2 0.1 0.05"/>

</geometry>

</visual>

<collision>

<geometry>

<box size="0.2 0.1 0.05"/>

</geometry>

</collision>

</link>

<joint name="left_ankle" type="revolute">

<parent link="left_shin"/>

<child link="left_foot"/>

<origin xyz="0 0 -0.4"/>

<axis xyz="1 0 0"/>

<limit lower="-0.5" upper="0.5" effort="20" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<gazebo>

<sensor name="left_foot_contact_sensor" type="contact">

<pose frame="left_foot">0 0 0 0 0 0</pose>

<contact>

<collision>left_foot</collision>

<surface>

<friction>

<ode>

<mu>1.0</mu>

<mu2>1.0</mu2>

</ode>

</friction>

</surface>

</contact>

</sensor>

</gazebo>

<!-- ================= RIGHT LEG ================= -->

<link name="right_thigh">

<inertial>

<mass value="3"/>

<inertia ixx="0.05" iyy="0.05" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<cylinder radius="0.05" length="0.4"/>

</geometry>

</visual>

<collision>

<geometry>

<cylinder radius="0.05" length="0.4"/>

</geometry>

</collision>

</link>

<joint name="right_hip" type="revolute">

<parent link="pelvis"/>

<child link="right_thigh"/>

<origin xyz="0 -0.15 -0.075"/>

<axis xyz="1 0 0"/>

<limit lower="-1.0" upper="1.0" effort="50" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<link name="right_shin">

<inertial>

<mass value="2"/>

<inertia ixx="0.04" iyy="0.04" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<cylinder radius="0.04" length="0.4"/>

</geometry>

</visual>

<collision>

<geometry>

<cylinder radius="0.04" length="0.4"/>

</geometry>

</collision>

</link>

<joint name="right_knee" type="revolute">

<parent link="right_thigh"/>

<child link="right_shin"/>

<origin xyz="0 0 -0.4"/>

<axis xyz="1 0 0"/>

<limit lower="0" upper="1.5" effort="50" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<link name="right_foot">

<inertial>

<mass value="1"/>

<inertia ixx="0.01" iyy="0.01" izz="0.01"

ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry>

<box size="0.2 0.1 0.05"/>

</geometry>

</visual>

<collision>

<geometry>

<box size="0.2 0.1 0.05"/>

</geometry>

</collision>

</link>

<joint name="right_ankle" type="revolute">

<parent link="right_shin"/>

<child link="right_foot"/>

<origin xyz="0 0 -0.4"/>

<axis xyz="1 0 0"/>

<limit lower="-0.5" upper="0.5" effort="20" velocity="1.0"/>

<dynamics damping="10.0" friction="1.0"/>

</joint>

<gazebo>

<sensor name="right_foot_contact_sensor" type="contact">

<pose frame="right_foot">0 0 0 0 0 0</pose>

<contact>

<collision>right_foot</collision>

<surface>

<friction>

<ode>

<mu>1.0</mu>

<mu2>1.0</mu2>

</ode>

</friction>

</surface>

</contact>

</sensor>

</gazebo>

<!-- ================= TRANSMISSIONS ================= -->

<transmission name="left_hip_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="left_hip">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="left_hip_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<transmission name="left_knee_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="left_knee">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="left_knee_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<transmission name="left_ankle_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="left_ankle">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="left_ankle_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<transmission name="right_hip_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="right_hip">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="right_hip_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<transmission name="right_knee_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="right_knee">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="right_knee_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<transmission name="right_ankle_trans">

<type>transmission_interface/SimpleTransmission</type>

<joint name="right_ankle">

<hardwareInterface>hardware_interface/EffortJointInterface</hardwareInterface>

</joint>

<actuator name="right_ankle_motor">

<mechanicalReduction>1</mechanicalReduction>

</actuator>

</transmission>

<!-- ================= GAZEBO ROS CONTROL ================= -->

<gazebo>

<plugin name="gazebo_ros_control" filename="libgazebo_ros_control.so">

<robotNamespace>/biped</robotNamespace>

<robotSimType>gazebo_ros_control/DefaultRobotHWSim</robotSimType>

</plugin>

</gazebo>

<gazebo>

<self_collide>false</self_collide>

</gazebo>

</robot>

## 3. Launch file to spawn robot into the room

nano launch/spawn_biped.launch

<launch>

<!-- Room -->

<include file="$(find biped_simulation)/launch/room.launch"/>

<!-- Robot description -->

<param name="robot_description"

command="$(find xacro)/xacro $(find biped_simulation)/urdf/biped.urdf"/>

<!-- Spawn robot -->

<node name="spawn_biped" pkg="gazebo_ros" type="spawn_model"

args="-urdf -param robot_description -model biped -x 0 -y 0 -z 0.45"/>

<!-- Load controllers -->

<rosparam file="$(find biped_simulation)/config/controllers.yaml" command="load"/>

<!-- Controller spawner -->

<node name="controller_spawner"

pkg="controller_manager"

type="spawner"

args="joint_state_controller

left_hip_controller left_knee_controller left_ankle_controller

right_hip_controller right_knee_controller right_ankle_controller"

output="screen"/>

</launch>

## 4. Create Robot Movement Script

nano scripts/stage1_controller.py

#!/usr/bin/env python3

import rospy

import math

from gazebo_msgs.srv import SetModelState, GetModelState

from gazebo_msgs.msg import ModelState

class Stage1Controller:

def __init__(self):

rospy.init_node("stage1_controller")

self.robot_name = "biped"

self.obstacle_name = "moving_block"

self.state = "HOME"  # Initial state is HOME

self.home = (0.0, 0.0)  # Home position

self.goal = (3.5, 1.0)  # Goal point (was wall_x=3.5)

self.step = 0.03     # Step size for movement (adjustable)

self.safe_dist = 1 # Minimum distance to obstacle (adjustable)

self.sidestep_direction = 1.0  # 1 for right, -1 for left; alternate to prevent drift

self.tolerance = 0.6 # Tolerance for when the robot is close to the home position

rospy.wait_for_service("/gazebo/set_model_state")

rospy.wait_for_service("/gazebo/get_model_state")

self.set_state = rospy.ServiceProxy("/gazebo/set_model_state", SetModelState)

self.get_state = rospy.ServiceProxy("/gazebo/get_model_state", GetModelState)

self.rate = rospy.Rate(20)

def distance(self, a, b):

"""Calculate Euclidean distance between two points"""

return math.sqrt((a[0]-b[0])**2 + (a[1]-b[1])**2)

def run(self):

"""Main control loop for robot navigation"""

while not rospy.is_shutdown():

robot = self.get_state(self.robot_name, "world")

obs = self.get_state(self.obstacle_name, "world")

rx, ry = robot.pose.position.x, robot.pose.position.y

ox, oy = obs.pose.position.x, obs.pose.position.y

rospy.loginfo("State: %s, Robot position: (%f, %f)", self.state, rx, ry)

if self.state == "HOME":

rospy.loginfo_once("STATE: GO_TO_GOAL")

self.state = "GO_TO_GOAL"

elif self.state == "GO_TO_GOAL":

if self.distance((rx, ry), self.goal) < self.tolerance:  # Reached the goal

rospy.loginfo_once("Reached the goal. Returning home...")

self.state = "RETURN_HOME"

else:

self.move(rx, ry, ox, oy, forward=True)

elif self.state == "RETURN_HOME":

if self.distance((rx, ry), self.home) < self.tolerance:  # Close enough to home

rospy.loginfo_once("Reporting back from reaching the origin!")

rospy.loginfo("Robot reached the starting point (origin).")

self.state = "DONE"

else:

self.move(rx, ry, ox, oy, forward=False)

elif self.state == "DONE":

rospy.loginfo_once("TASK COMPLETED")

self.halt_robot()

self.rate.sleep()

def move(self, rx, ry, ox, oy, forward=True):

"""Move the robot with obstacle avoidance and goal navigation"""

target = ModelState()

target.model_name = self.robot_name

target.reference_frame = "world"

dist_to_obs = self.distance((rx, ry), (ox, oy))

if dist_to_obs < self.safe_dist:

rospy.loginfo_once("Obstacle detected, sidestepping.")

# Sidestep in alternating direction to prevent constant drift

target.pose.position.y = ry + self.sidestep_direction * 0.03

target.pose.position.x = rx

# Alternate direction for next time

self.sidestep_direction *= -1.0

else:

if forward:

# Move forward towards the goal

dx = self.goal[0] - rx

dy = self.goal[1] - ry

dist = self.distance(self.goal, (rx, ry))

if dist > self.step:

target.pose.position.x = rx + self.step * (dx / dist)

target.pose.position.y = ry + self.step * (dy / dist)

else:

# Close enough, snap to goal

target.pose.position.x = self.goal[0]

target.pose.position.y = self.goal[1]

else:

# Return home: move directly towards home position

dx = self.home[0] - rx

dy = self.home[1] - ry

dist = self.distance(self.home, (rx, ry))

if dist > self.step:

target.pose.position.x = rx + self.step * (dx / dist)

target.pose.position.y = ry + self.step * (dy / dist)

else:

# Close enough, snap to home

target.pose.position.x = self.home[0]

target.pose.position.y = self.home[1]

# Handle potential left wall (assuming there's one around x=0)

if target.pose.position.x < 0.0 + self.step:

# Divert if hitting left boundary/wall

target.pose.position.x = 0.0 + self.step

target.pose.position.y = ry + self.sidestep_direction * 0.03

self.sidestep_direction *= -1.0

target.pose.position.z = 0.45  # Ensure the robot's height is correct

target.pose.orientation.w = 1.0

self.set_state(target)

def halt_robot(self):

"""Continuously force the robot into a perfect standing pose at home with zero velocity.

This keeps the robot locked in place even while the Gazebo simulation is still running."""

target = ModelState()

target.model_name = self.robot_name

target.reference_frame = "world"

# Lock to home position (standing pose)

target.pose.position.x = self.home[0]

target.pose.position.y = self.home[1]

target.pose.position.z = 0.45

target.pose.orientation.w = 1.0

# Zero velocities - this is what stops the "flying off"

target.twist.linear.x = 0.0

target.twist.linear.y = 0.0

target.twist.linear.z = 0.0

target.twist.angular.x = 0.0

target.twist.angular.y = 0.0

target.twist.angular.z = 0.0

self.set_state(target)

if __name__ == "__main__":

controller = Stage1Controller()

controller.run()

## 5.launch

nano launch/stage1.launch

<launch>

<node name="stage1_controller"

pkg="biped_simulation"

type="stage1_controller.py"

output="screen"/>

</launch>

### Phase 3 — Moving block

We will:

mkdir -p moving_block

## 1.Create model

nano models/moving_block/model.sdf

<?xml version="1.0" ?>

<sdf version="1.6">

<model name="moving_block">

<static>false</static>

<link name="block_link">

<pose>0 0 0.25 0 0 0</pose>

<collision name="collision">

<geometry>

<box>

<size>0.4 0.2 0.5</size>

</box>

</geometry>

</collision>

<visual name="visual">

<geometry>

<box>

<size>0.4 0.2 0.5</size>

</box>

</geometry>

<material>

<ambient>0 0 1 1</ambient>

<diffuse>0 0 1 1</diffuse>

</material>

</visual>

<inertial>

<mass>20.0</mass>

<inertia>

<ixx>0.5</ixx>

<iyy>0.5</iyy>

<izz>0.5</izz>

</inertia>

</inertial>

</link>

</model>

</sdf>

## 2.Create movement script

nano scripts/move_block.py

#!/usr/bin/env python3

import rospy

import math

from gazebo_msgs.srv import SetModelState

from gazebo_msgs.msg import ModelState

rospy.init_node("moving_block_controller")

rospy.wait_for_service("/gazebo/set_model_state")

set_state = rospy.ServiceProxy("/gazebo/set_model_state", SetModelState)

rate = rospy.Rate(50)

t0 = rospy.Time.now().to_sec()

while not rospy.is_shutdown():

t = rospy.Time.now().to_sec() - t0

state = ModelState()

state.model_name = "moving_block"

state.reference_frame = "world"

state.pose.position.x = 2.0 + 0.6 * math.sin(0.5 * t)

state.pose.position.y = 0.0

state.pose.position.z = 0.25

state.pose.orientation.w = 1.0

try:

set_state(state)

except rospy.ServiceException as e:

rospy.logerr(e)

rate.sleep()

## 3.Create launch

nano launch/moving_block.launch

<launch>

<node name="move_block"

pkg="biped_simulation"

type="move_block.py"

output="screen"/>

</launch>

Rebuild your workspace:

```

cd ~/catkin_ws

catkin_make

source devel/setup.bash

```

Restart the simulation:

```

roslaunch biped_simulation spawn_biped.launch

roslaunch biped_simulation moving_block.launch

roslaunch biped_simulation stage1.launch